|  | TSW Controller Utility Profile ¦ TCA Airbus |  |
| --- | --- | --- |
|  | HR1500 |  |
| From https://steamcommunity.com/sharedfiles/filedetails/?id=1948213106 |  |  |
| AFB ⏻ Signallights tail ¦ off ¦ head Horn LZB/PZB Acknowledge LZB/PZB Release 2 L Train brake Throttle AFB Direct brake Release Hold Apply 140/160 km/h 0 km/h Reverser Sand SiFa reset AWS ⏻ Vigilance ⏻ Open / Close ←L Doors R→ Master key Recommended initial position of lever / knob |  |  |

0:	-
  BTV: 	Reverse
  LO: 	Off
  2: 	Neutral

3: 	Forward

HI: 	-

| Cab setup | Cleared to depart |
| --- | --- |
| - Insert master key - Set reverser to neutral - Open doors - Turn on safety system (AWS) - Turn on vigilance system - Set headlights and taillights | - Check signal - Set reverser to forward - Release brakes - Apply power |

| DLC: | https://store.steampowered.com/app/3879910/Train_Sim_World_6_Expert_DB_BR_145_Loco_AddOn/ |  |  |
| --- | --- | --- | --- |
| Cold and dark |  |  |  |
| - Ensure the locomotive is under overhead wires. - Ensure at least one brake is applied. This is indicated by a red square with a black dot. - Enter the locomotive through Cab 1 and enable the battery. - Ensure that all circuit breakers on this backwall are closed. - Check the normal position of all the switches on the backwall. - Set headlights as required. - Set brake mode as required (R for passengers/light, P/G for freight depending on weight). - Check that the Auxiliary Controls are off (side of cab for shunting / auxiliary operation). - Insert the reverser and set it to V (Forward). - Set Signal & Instrument Lights to on. - Ensure AFB is off. - Release the brakes until C-pressure is at 0.5 bar or lower. |  | - Ensure the Parking Brake is on (light illuminated). - In the engine room, ensure every circuit breaker is closed. - Ensure PZB, LZB and Sifa are in their normal position. - Open the Sifa Cut-Off Valve. - Ensure the Pantograph Cut-Off Valve is open. - Ensure the Driver’s Brake Cut-Off Valve is closed. - Walk to Cab 2 and check that all circuit breakers are closed. - Check the normal position of all the switches on the backwall. - Set headlights as required. - Ensure the Direct Brake is set to the “Hold” position. - Test the Horn Pedal. - Ensure that the Air Compressor Switch is on. - Set Signal & Instrument Lights to on. - Return to Cab 1. - Raise the Pantograph and close the Main Circuit Breaker. - Enable the Air Compressor |  |

| Brake System Preparation |  |
| --- | --- |
| - Ensure that the Brake Pipe stays at 5 bar. - Move the Reverser to M (Neutral) - Ensure that the Brake Pipe stays at 5 bar. - Move the Reverser to V. - Decouple the Dynamic Brake. - Reduce Brake Pipe Pressure by 0.8 bar with the Train Brake and monitor the Brake Pipe gauge. - Release the brakes. - Go to the “Prüfen” page. [W → Prüfen] - Press SoftKey 5 “BrR 2 Ein” and await the HSM change. - Decouple the Dynamic Brake. - Reduce Brake Pipe Pressure by 0.8 bar with the Train Brake and monitor the Brake Pipe gauge. - Release the brakes. | - Apply the Direct Brake and monitor the C-pressure gauge. - Release the Direct Brake and monitor the C-pressure gauge. - Perform a PZB/LZB test and a Sifa test. - Apply the Direct Brake. - Release the Parking Brake. - Turn the Machine Room Lights and Cabin Lighting off if necessary. - Test the Horn Pedal. - Perform a rolling test: - Release all brakes. - Apply a small amount of power. - Roll for a few metres. - Reduce power to 0. - Apply a medium amount of Direct Brake force and stop. |